By Joseph Padfield

November 1999

Hamilton Kerr Institute

The Procedure for Examining Paintings under IR

And Mosaicing IR images, using the

SUN Computer.

Table of contents:

1.	Introduction:	3

2.	Set-up:	3

3.	Procedure for simple IR examination of a painting:	4

4.	Procedure for capturing IR images:	6

4.1.	Turning on the Computer:	6

4.2.	Creating your own folders:	7

4.3.	Opening the Infra-red imaging software:	8

4.4.	Capturing images:	9

Capturing new images:	10

4.5.	Saving and Naming Images:	12

Preparation for Saving Images:	12

4.6.	Correcting your images for irregularities in the Tube.	14

5.	Procedure for Mosaicing IR images:	15

Removing Entries from Columns:	15

5.1.	Loading images:	16

5.2.	Looking it at images:	16

5.3.	Mosaicing:	16

Selecting and removing mosaicing points:	17

Mosaicing:	17

Common, Simple Mosaicing problems:	18

5.4.	Workspaces and Their Structure:	19

Structuring Your Workspace:	19

5.5.	Problems with Mosaics:	21

5.6.	Balancing the contrast of the image:	22

5.7.	Converting your finished image:	23

5.8.	Transferring your image and printing:	23

6.	Continuing Previous Work:	23

7.	Process Summary:	24

Selecting and removing points for mosaicing:	25

Mosaicing:	25

8.	Common General Problems:	27

9.	Shutting Down The Sun Computer:	27

## Introduction:

This set of instructions is intended to explain the basics instructions required to, turn on and use the SUN computer, to capture images from the IR camera and then to mosaic them together.  The instructions manuals supplied with the IR software go into much greater detail and even explain how the program actually works, so if you require more information than is supplied in this report please consult the software instructions manuals.

This report will begin with a description of the equipment available, followed by a description of how paintings can be simply examined under IR  (based on the notes produced by B. A. Campano in June of 1998).  Finally the procedures required for the mosaicing of IR images using the SUN computer will be discussed.

## Set-up:

The equipment set up for the infrared examination of paintings is as follows:

- One wall mounted easel to which paintings can be secured

- One infrared camera mounted with two infrared lamps on an adjustable platform and support frame.

- One computer stand.

- One SUN computer comprising of; a monitor on the left of the top shelf of the computer stand and four light grey boxes on the second shelf of the computer stand.  There is one wide box, (the main part of the computer), on the left and three smaller boxes, stacked one on top of each other, on the right of the shelf.

- One dark grey metal box, with a large dial, on the bottom left of the computer stand; this is the power source for the IR lamps.

- One Hamamatsu camera control unit, which is the slim grey box on the top shelf of the computer stand, to the right of the computer monitor.

- Finally their are three black and white monitors, two mounted on a stand to the left of the easel and the final one positioned to the right of the computer monitor on top of the Hamamatsu camera control unit.

- In addition, there are a number of cables connecting the various parts of the computer, monitors and the camera, generally these should all be left alone.  The only exceptions to this are the four power cables, (which can be plugged into the wall sockets on the wall behind the computer stand) and the network cable.

- The network cable is the thin grey/white cable coiled up on the right of the bottom shelf of the computer stand, this is used to connect the SUN computer to the PC in the photographic studio, but this will be discussed later.

## Procedure for simple IR examination of a painting:

NB: If you are going to be using the computer it is a good idea to turn it on first as it can take a while to warm up, see section 4.1.

Find your painting: It is annoying to set everything up and then not be able to get your hands on the painting.

Place the painting on the wall-mounted easel and secure it in place: For simple examinations the exact positioning of the painting is not too important.  However, in order to produce a good mosaic the painting needs to be placed parallel to the plane of the camera lens, in an attempt to ensure that as the camera moves it remains the same distance from the surface of the painting.

Plug in the four power leads: These leads will be behind the computer stand and can be plugged into the wall just to the right of the monitor stand.

Turn on the monitors: i.e. the two monitors on the stand near the wall and the smaller monitor on the computer stand.  Fell free to play with the brightness and contrast to improve the image on each of the monitors.

Turn on the IR lamps: The power for the IR lamps is supplied from the grey metal box on the bottom left of the computer stand.  There is a small switch and a large dial.  Flip the switch and then turn the dial to between 40 and 60 V.  If the camera is very close to the surface of the painting or very far away you can adjust the power of the IR lamps as appropriate by turning the dial up or down.  When not in use the IR lamps can be switched off by turning the dial back down and clicking the switch to off.

Turn on the camera: The camera is controlled using the Hamamatsu camera control unit, which is to the right of the computer monitor on the top shelf of the computer stand.  When the unit is switched on the small green light will come on.  NB: The tube in the IR camera is EXPENSIVE and should not be left on for extended periods of time if it is not being used, so just switch it off when you are not using it.   NB: If the Lens cap is on the lens it can be removed at this stage and it can be left on the platform just next to the camera.

Focusing the camera: This can simply be achieved by carefully rotating the end of the lens, in either direction until the image appears clear.  You will have to check the focusing of the camera every time you alter the distance between the camera and the surface of the painting.

Adjusting the image produced by the camera: To adjust the image watch one of the monitors as you slowly vary the positions of the following three dials on the Hamamatsu control unit.  The sensitivity dial should originate between the twelve to one o’clock positions.  The gain and the offset dial should both originate at the twelve o’clock position.  Gradually, with one hand on the gain dial and the other on the offset dial, turn the dials away from each other until the image is satisfactory (the gain dial is turned slightly to the left, then the offset is turned slightly to the right, etc.)  Often this will result in the gain at eleven o’clock and the offset at two o’clock.  While adjusting these dials be sure to keep an eye on the red indicator light to the right of the sensitivity dial.  Should this red light come on the camera is experiencing burn and the sensitivity dial should immediately be turned down (to the left) until the red light goes out.  NB: if the image is only visible when the red light is on, the brightness of the monitors needs to be increased.  The sensitivity dial can be carefully adjusted left or right in order to improve the captured images, but again do not allow the red light to be left on.

Positioning the camera stand: The camera stand can be moved back and forward to increase or decrease the area of the painting seen on the monitor.  Obviously as you move the camera closer to the painting it can detect greater detail.  When the system is being used to produce a mosaic the camera stand has to be positioned horizontal to the wall.  This can be achieved by ensuring that the distance between both of the front wheels and the raised section of floor is the same.  The distance between the wheels and the raised section of floor can be simply measured it can be set using some of the wooden blocks which are normally found down to the left of the easel.  Once the stand is positioned properly the wheels can be locked, by turning the small orange cog, anti-clockwise, on each of the wheels.  While positioning the camera stand it is good to have the actual camera positioned in the middle and pointed at the middle of the painting or area of interest, for instructions on how to move the camera see below.

Adjusting the position of the camera: The platform to which the camera and the IR lamps are secured can be moved up/down and left/right.  By releasing the locking levers the position of the platform can be adjusted by a specific number of notches.  Thus once the painting and the camera stand has been secured in position the camera can be moved around and returned to the same position by counting the number of notches it has been moved, in any of the four directions.

Shutting down: If you have been using the computer, NEVER just switch it off, it needs to be shutdown properly.  At this time only Joe and Chris can shut the computer down so leave it on a let one of them know.

The camera control, the three monitors and the IR source can all be switched off, however when unplugging the power leads make sure you do not unplug the computer until it has been properly shutdown.  If in doubt leave all of the plugs connected.

## Procedure for capturing IR images:

Once a painting has been placed in position and the camera has been set up, as described in section 3, the image capturing and mosaicing can begin.

Ensure that the computer power cable is plugged in, this should already have been done when the camera and monitors were set up but check just in case.  If the computer is not plugged in simply plug in all four of the power leads, they will be lying on the floor somewhere between the back of the computer stand and the plug sockets on the wall just to the right of the monitor stand.

NB: When you are capturing images to produce a mosaic all the images required need to be captured at the same time, to ensure that they will match with each other.  So plan your time well !!

### Turning on the Computer:

NB:  Care must be taken when turning the computer on or off.  Follow the instructions given.  However it should be noted that the computer is happy to be left on all day so you do not have to rush to turn it off, as turning it back on always takes a good few minutes.

There are five parts to the computer, which can be switched on, or off.  Firstly the monitor, the on/off switch actually on the monitor is normally left in the on position so that it will just come on when the computer is turned on, but check if there are any problems.  Next there are the four computer boxes on the second shelf of the computer stand, the larger main section and the three smaller section stacked to the right.  The switches for each of these boxes can be found around the back.  NB: when starting always switch on the three smaller boxes first and then the larger one, if you do it the other way round you can confuse the poor little computer.  Once all four boxes have been switched on the computer will boot up.  This can take a little while, (several minutes), so please be patient.

During the booting up procedure the computer will ask you to enter your ‘user name’, unless you have been given a unique user name by Chris you will need to use the standard user name which is: general  Once you have typed this in, press the return key. The computer will then ask for your password, the password for the ‘general’ user name is fun@sun  Again once you have typed this in, press the return key.  The computer will now finish booting up, again please be patient.

The computer will open its windows program in black and white, with little dolphins on the background.  Three windows will open: a ‘cmdtool’, a ‘cmdtool (console)’ and a copy of ‘File Manager’.

NB It is important to note that when using the SUN computer, though you may have various windows open at the same time, you can only interact with one at a time.  You can indicate which window you wish to interact with, simply by moving the mouse pointer or arrow over the window you wish to use.

NB: At this stage it should be noted that the mouse used with the SUN computer has three buttons.  While using the mouse these buttons are used in three ways; a single click, a double click, or they can be held down for a while and then released.  During this report a four-letter abbreviation will be used to indicate how the mouse should be used.  The first letter will indicate whether you should click (C), double click (D) or hold (H) the mouse button the second letter will indicate which button to use left (L), middle (M) or right (R) and the final tow letters ‘MB’ just stand for mouse button.  For example CLMB means to click the left mouse button once.

### Creating your own folders:

Create your own folder or directory using the file manager: CRMB on the heading ‘File’ in the ruler at the top of the file manager window and then CRMB on the entry ‘Create Folder’, in the drop down menu which appears.

A new folder will appear in the file manager, with the words ‘new folder’ highlighted underneath.  If you begin typing immediately you can give the folder your own name, i.e. type in the new name and then press return.

If the name is not highlighted or you wish to re-change the name of the folder then simply CLMB once over the name and it will become highlighted, as described above, if you CLMB twice you can alter the existing name rather than replacing it when you begin typing.

It will be good to produce your own folder, as described and then DLMB on the actual picture of the folder to enter it, NB: the path at the top of the file manager will change to include your new folder.

Now within your own folder it is good to produce separate new folders, following the same procedure as before, for each piece of work which is conducted i.e. each painting which is examined under IR.

### Opening the Infra-red imaging software:

While the mouse arrow is over the background. CRMB and a small menu will appear.

CRMB on the line marked ‘Programs’ and then again on ‘Infrared System’ in the second menu which has appeared.  This will open the program that is used to capture and mosaic images; this may take a while please wait.

The widow that appears will have ‘ip.binary.s26’ at the top; this is the main window for controlling the IR program.  From here on it will be referred to as the ‘ip window’. Increase the size of the ‘ip window’ to cover most of the screen, this is not required but it will give you more area to work with.

At this point you will need to turn on the camera and set it up as described in section 3, if it has not already been done.

On the ruler at the top of the ip window, HRMB on an area free from words and then move the arrow down to ‘infrared’ on the menu which will appear then let go of the mouse button.

THIS WILL SIMPLIFY THE WINDOW. DON’T WORRY, THE WORDS ARE MEANT TO DISAPPEAR.

The ip window with the infrared menu list, will have the following headings: [File] [Edit] [Balance] [Dialogs] [Grab] [Mosaic] and [Shrink].  By CLMB once over these headings menus will be produced to perform commands or move through to further sub menus, which can again be accessed by simply CLMB over the entries in the new menus.  To simplify these instructions the commands performed within the IR software will be indicated as follows:

[Window] [ruler entry: menu entry: etc.]

For example: [ip Window] [File: New: Column]

This will indicate that, from the ip Window ruler bar you should CLMB once on the ‘File’ heading and then CLMB once on the ‘new’ entry in the drop down menu and then finally CLMB once on the ‘column’ entry in the sub menu.

It should be noted at this point that this piece of software is quiet complex and is capable of doing many things.  The following instructions only cover the basics of capturing and mosaicing images, for further information please consult the software manuals.

NB: In order to organise your work, while using the IR software, the programmer has designed a column system.  All information, which the program is considering, is listed in smaller special windows within the ip window called columns. The columns produced by this program are each given a letter, the first column being ‘A’, the next ‘B’ and so on.  Entries within these columns, such as video windows, images mosaicing instructions etc. included an accessing button, the name and a description of the entry. In order to access any entries in a column simply DLMB over the button on the left of the entry.  Large number of columns can be opened, using the command [ip Window] [File: New: Column], so that you can organise your images and calculations. Further information regarding columns will be discussed during the mosaicing instructions, see section 5.

### Capturing images:

- [ip Window] [File: New: Video]

This will add a video option to your currently selected column.  Then DLMB on the video button produced in the column, to open the video window.  This will show the still image which was seen through the camera when the [New: Video] command was made.

NB: the image captured has been cropped at the edges to remove the excess curvature that occurs at the edges of the full image.  Again please consult the manual if you require further information regarding this.

Magnifying your images and Moving around Viewing Windows:

When you have opened an image or video window the magnification of the image can be altered, so that you can zoom in on details or move back to see the entire image, when it is quite big.  To alter the magnification of an image use the following command:

- [Video (or Image) window][View:Magnification:*]

Where ‘*’ equals the desired magnification.  It is probably good for you to try this now so that you will be familiar with it later.

NB: Sometimes for some obscure reason the magnification will not change even though you have asked it to.  In this case try changing it to a completely different magnification and then back again.  If all else fails, try swearing at the computer, it won’t do anying, but you might feel better.

When you are looking at big images, or smaller images at higher magnifications, all of the image will not be visible in the window at the same time.  In order to see the entire image you can either increase the size of the window or you can use the sliders at the edge of the window to move the image so that different areas of it can be seen through the window.

#### Capturing new images:

The video window can be updated, i.e. new images captured by holding down the control key and pressing ‘C’, this command is normally denoted as CTRL-C.   Each time CTRL-C is pressed the image currently being sent from camera is captured and displayed in the video window.  The camera can then be moved between dark areas and light areas and the camera controls can be altered as described previously to produce an acceptable image.

The camera is then positioned, i.e. moved towards or away from the painting, focused and secured in position, so that the camera is capturing sufficient detail.  NB: The closer the camera is to the surface of the painting the greater the detail captured, but the larger the number of images required to cover the area of interest.

NB: If an entire painting is being captured think about the number of images that will be needed.  Any more than about 80 can lead to problems if they are processed all at once.  With larger numbers of images the mosaic can be put together in sections, e.g. quarters or thirds.  Remember also that the output of the printer can only produce A3 images so this limits the resolution of the output.

- Move the camera so that an image of the upper left corner of the painting or area of interest can be captured.

Open a new video window, i.e. [ip Window] [File: New: Video].  This again adds a video option to the column and by DLMB on the new button produced on the column the second video window will be opened.  Then move the two video windows so that they can both be seen, left and right of each other is good.

- Move the camera to the right two notches and capture a new image.

Check the overlap between the first and the second image.  It is good to ensure an overlap of about 1-2 cm.  If the overlap is to big or too small the number of notches moved can be increased/decreased or the camera can be moved further away from the painting.  The position of the camera or the number of notches moved can be altered and new images captured.  Once a comfortable overlap has been achieved close one of the video windows and then move the camera back so that it can capture the top left corner of the area of interest once more.

The stand is designed such that once the correct overlap has been achieved for the left/right directions, the up/down overlap will also be correct, thus it does not need to be checked separately.

NB: It should be noted that if the camera is left pointed at one thing the image will begin to be imprinted in the camera so that when the camera is moved a ghostly image on the previous image will remain for a while.  This phenomenon is called ‘Ghosting’.  Ghosting needs to be avoided or compensated for when capturing images to ensure that the captured images are correct.  Therefore, the camera should not be left in anyone place for more than a few seconds while you are capturing images, or you need to pause between each image you capture to ensure that the previous image has cleared from the cameras memory.  While you are not actually capturing images the ghosting problem is not such a great concern, but you should be aware of it.

### Saving and Naming Images:

#### Preparation for Saving Images:

- [Video Window] [File: Save].

This will open a save window associated with the video window.  Within this window there are several entries and these have been explained below.

- The ‘PATH’ denotes were on the computer the images will be saved.

- The ‘FILTER’ denotes the type of file that the computer will save the image as.

- ‘DIRECTORIES’: list the subdirectories present within your current folder.

- ‘FILES’: list the files present within your current folder.

DLMB on the name of your directory, the ‘PATH’ should then read /home/general/’X’ where X is your directory.   If you have created another subdirectory within this directory you will now have to DLMB on it, to change the ‘PATH’ to include it.

In the bar, entitled ‘selection’ at the bottom of the window  ‘untitled.v’ should be written.  The text in this box shows that name the computer will give to an image when you save it.  By CLMB over the bar a flashing cursor will appear next to the text and you can alter the name.   NB: the name you designate for your image must end in ‘.v’

TIP:  Plan your image names, it is good to name your images in rows so that you automatically know were each images goes in relation to the others.  Even if you do not want to mosaic a square or rectangular area it will simplify the mosaicing process if you simply capture a little more.  NB:  the capturing process is quite quick, thus capturing a few more images is not a serious problem.  Also you do not need to mosaic all the images you capture, but it is not really possible to go back and recapture individual images at a later date as they all need to be captured at the same time.

1           2            3

R1

R2

R3

gy

Figure 1: Schematic diagram, demonstrating how images can be named.

When naming your files start with some initial, from the name of the artist or the name of the painting, i.e. for goya you could use ‘gy’.  Then add ‘R’ followed by a number to denote the number of the row and then a number to denote how many images it is along from the left-hand side.  For example: The top left image above would be named: gyR1.1.v and the next one across would be gyR1.2.v and so on etc, see figure 1.

Begin capturing your images from the upper left corner, the process must be smooth and reasonably quick so that the camera is not left looking at the same image for than a few seconds, this avoids ghosting, thus need two people.  One person to move the camera and one to capture and save the images on the computer.  Also it is good to always capture your images from the left to the right, as this will help keep the naming process consistent.

If there is only one person then the camera lens could be covered between each image being captured to remove any ghosted image or you can simply wait a while between capturing images to ensure that the previous ghosting has faded.

However the best and quickest method is to use two people. Once you have everything set up and you are happy with the procedure it should not take too long to capture all of the images.  It only took about 30 minutes to capture 72 separate images required for one painting.

When using the naming format **R1.1.v   etc., if your click on the small square next to the heading ‘Increment filename on save’ in the save window the last number will automatically increase by 1 each time you save, i.e.  **R1.1.v  **R1.2.v   **R1.3.v etc.  This allows you to save entire rows of images without having to edit the filenames.  NB: remember to change to row number R1 R2 R3 etc. and put the last number back to 1 when you finish each row and move the camera back to the left and down to begin the next row.

Position the open video window with the save window next to it.  Set the filename as described before, ensure the camera is facing the top left corner of the painting or area of interest and then place a box over the end of the camera.  (The camera lens cap is not used as this would mean that you would have to refocus the camera when you remove it, which would be to slow).  This covering of the lens is designed to remove the ghosting described above.

Remove the box wait a couple of seconds and then press ‘CTRL-C’ to capture the image.  In order to then save the image, CLMB over the [OK] button in the save window. This will save your image and then alter the file name ready for the next image.

Move the camera to the right, by the decided number of notches, capture the new image, save it and then continue until the end of the row.  At the end of the row move the camera back to the left and then lower it, by the decided number of notches, to begin capturing the next row.  NB: the file name must be corrected at the beginning of each new row.

If the overlap area, between any of your images, is featureless, it is worth identifying a good tie-point and ensuring it is visible in both images, even if this means increasing the overlap for one of the joins.

### Correcting your images for irregularities in the Tube.

Once all of the images have been captured place the piece of light yellow card in front of the painting, this will be seen as a blank grey image on the monitors.  Turn the sensitivity dial on the camera control box to slowly to the right until the red light begins to flicker and then turn it back to the left until the red light just disappears, stop at this point.  This should produce an even grey image on the screen.  Capture this image and save it as ‘grey.v’.

CLMB on [cancel] in the save window, close the video window and then close the ip window.  This should return the screen to roughly the state it was when you turned the computer on.

Move the mouse pointer over to the ‘cmdtool’ window:

- Type in cd /home/general/**1/**2, (where **1 is the name of your own directory and **2 is the name of the subdirectory in which you have saved your images), and press return.

- Then type pwd and press return this command will verify that you are now in the right directory.  You can also type ls and then press return.  This will list all of the files that are in the current directory.

- If the ls command does not list all your files try make sure that you typed the cd /home/general/etc line in correctly.  If this still does not work please ask for help.

- If the ls command did list you images continue.

- Type in vips7.4 light_correct grey.v  *.v and then press return.  Please wait this can take a while if you have a lot of images.

- This command will then use the grey.v image to correct all of your captured images for any tonal errors due to the differences in the IR tube.

- The computer will now have produced a new version of each of your images.  They will all have the same names except that they will all begin with the prefix ic_  i.e. a file named gyR3.1.v will be duplicated as ic_gyR3.1.v

- These new ic_ images are then ready to be used to produce your mosaic.

You are now allowed to relax as you have completed the capturing and saving procedure and the fun of mosaicing can begin.

## Procedure for Mosaicing IR images:

This section is intended to explain the basic process through which a selection of images can be made into a mosaic.  The ip software is capable of doing various things and this document is not intended to explain the full usage of the software.  Feel free to read the provided manuals if you really want to learn more.

To mosaic images you only need the computer on, so you can leave the camera, monitors etc all off.  Please refer to section 4.1 for an explanation of how to turn on the computer.   The basics for mosaicing are explained first, but it is good to read section 5.4 before you actually start loading images and sticking them together.

Also you do not need to mosaic all of your images together at one time.

As long as you plan your work well you can carry out the mosaicing procedure whenever you have spare time.  However it can be easier to follow your work through if you can do it in one go.

- Turn on the computer as described in section 4.1.

- Open up the IR software as described in section 4.3.

As described previously, in section 4.3, the IR software opens up with an ‘ip window’ and once you have simplified the ruler, you are presented with the following headings: [File] [Edit] [Balance] [Dialogs] [Grab] [Mosaic] and [Shrink].  For the basic procedure of mosaicing a set of images you will only have to use [File], [Mosaic] and [Balance].

When you start to mosaic a set of images you are going to need to load them into the software and then begin to instruct the computer as to which pictures need to be stuck to which.  This is all done via the column windows, which were first introduced in section 4.3, but are now going to be discussed again.

#### Removing Entries from Columns:

At any time during your work you may want remove entries from your columns, this can be achieved as follows.

- First select the entry button or buttons which you want to delete.

Selecting and de-selecting entries in your columns can be done simply by CLMB over the button associated with the particular entry.  In order to select more than one entry at the same time you will need to hold down the ‘shift’ key while you CLMB on the entry buttons.

- To remove the selected entries, hold down the ‘control’ key and press ‘r’, i.e. press ‘Ctrl-R’.

### Loading images:

Images can be loaded into the software and added to a column using the following commands:

- [ip Window] [File: Load: Image]

This will open an image-loading window, as with the save window, discussed in section 4.5, the load window contains information relating to which directory you are looking into, which files are present and the type of file the program is currently trying to save.  Once you have made sure that you are in the right directory your images can be loaded into your column, simply by DLMB on the name of the image in the file list.  NB: You may want to move the load window so that you can see the column into which you are loading the images, this way you can see when they appear.  You can often DLMB the image name correctly but the image will not be loaded into the column, this is often to do with the speed at which you click the mouse buttons.  No not worry about this, just make sure each of the images you are wanting to load appears in the column as you DLMB on the image names in the loading window.

NB: If you are loading a selection of files ensure that the ‘pin me up’ box has been ticked.  This simply stops the load window from closing each time you load an image.

Once you have finished loading the images you require at that point, CLMB on ‘cancel’ in the load window to close it.

### Looking it at images:

Once you have loaded images into your column you can view them in an image window simply by DLMB over the button to the left of the specific image entry in the column.  To make the mosaicing process more straight forward you may want to play with the magnifications of the images, ( [image window][View:Magnification:*] )

### Mosaicing:

This actual process of mosaicing is quite straight forward; images are stuck together in pairs to produce larger images which are then stuck together in pairs and so on.

To mosaic two images together open viewing windows for both of the images and move them so that you can see both images at the same time.  NB: it is often easiest if you orientate the images so that the edges you wish to overlap are next to each other.  Next you need to help the computer by giving it a place to start when it tries to stick to two images together.

#### Selecting and removing mosaicing points:

Look at the two images and find a point, which is present on both; this can be a dot, the end of line, a patch of white, etc. it does not matter as long as it is present on both images.  Once you have found your point move the mouse arrow to your chosen point in the first image and CRMB, then do the same for the second image.  (NB: the order in which the points are chosen does not matter as the computer can work out which is to go were).  When you CRMB over your points a small cross will appear with a nametag.  These selected points are called regions and they are entered into you current column as with the images, with a button, description, etc.

If you find you have placed the points in the wrong place they can be moved by HLMB over the nametag and just moving it to the position you require.

NB: There is a bug/error in the program, which sometimes makes the nametag disappear when you try to move it.  In this case the nametag has not been deleted but just moved to another area on the image.  Here it is a good idea to alter the magnification of the image so that it all visible in the window and try to find your point and then relocate it.  If all else fails you can simply produce two new points, but this can begin to clutter up your columns so try to find the tag first.

Selected points/regions can be deleted either by removing the entry from the column or by DLMB over the actual nametag, this produces a small window with information relating to the selected point.  One of the options in this window allows you remove the point.

#### Mosaicing:

Once you have selected your overlap points the two images can be mosaiced using one of the following two commands:

- [ip window][Mosaic: Left_right_mosaic]

- [ip window][Mosaic: Top_bottom_mosaic]

Your choice between these two commands is obviously dependent on whether the images are to go together top and bottom or left and right.

Either of these commands will cause the computer to stick the two images together and enter the resultant image as the next entry in your column.

Due to the mathematics used by the IR software, each join will on average cause a positioning error of about 0.5 pixels, even if your input images contain no geometric distortion. If there are distortions in your input images (there usually are distortions in infrared images), then errors can be as much as 1{2 pixels per join. These errors accumulate, as the number of images you join becomes larger.

If you join a strip of 10 images together with Mosaic left right, on average you can expect a total error of about 5 pixels. If you join two strips like this together top-bottom, you can therefore expect a mismatch of about 2.5 pixels at each end of the join.

You can minimise the effect of these errors if you assemble your images differently. Suppose you have a 10 by 10 mosaic to build. Instead of making and joining 10 strips of 10 images each, make four 5 by 5 sub-mosaics (one for each quadrant) and then join these four quadrants together. The errors will now be more evenly spread over the image and therefore will be less visible.

#### Common, Simple Mosaicing problems:

- The computer will sometimes complain that there is insufficient overlap to mosaic your two images.

This can often be sorted by moving your points to a different overlap position.  It is good to have the points roughly the same distance from the edge on both images.  Also if you are trying to mosaic bigger images, which are themselves composed of many smaller images, the edges of the images may not be straight so you need to ensure that you have sufficient overlap right along the edge of the image.

This error message has also been produced in another situation.  If you have tried the above solution without success check the co-ordinates of the region.  If the numbers are too high and the computer does not register the co-ordinates when you move the mouse arrow across the image you will need to clone the image.  This can be done in any of your columns but it might be good to open a new column just to fix this problem.  Select the column into which you would like the new version of the image to appear.  HRMB over the button of the specific image and then choose ‘clone item’ from the menu which appears.  You should then be able to carry on mosaicing your images using the cloned image rather than the original.

- The computer may inform you that it needs 2 arguments rather than one.

This just means that you have accidentally selected one of the entries in your column and you need to de-select it or select the other region if you had deliberately tried to select your regions.  Selecting and de-selecting entries in your columns can be done simply by CLMB over the button associated with the particular entry.  In order to select more than one entry at the same time you will need to hold down the ‘shift’ key while you CLMB on the entry buttons.

- Strange images or images with lines or marks can be produced if your selected points do not match correctly, or if you choose the wrong command, either left_right or top_bottom mosaic.

If your points do not match up properly you can just move them as described before and the computer will automatically correct the mosaiced image.  If you have already closed the images so that you can no longer see the positions of your points you can just re-open them by DLMB over the button next to the entries for the two regions in question.

If you have used the wrong command you will have to remove the incorrect image and them use the correct command.  In this situation the computer will them just mosaic the last two regions in your selected column, thus producing the correct image.

### Workspaces and Their Structure:

All of your columns, with the commands and images are known as a ‘workspace’, like any other working area unless you keep it ordered you will soon loose track of what you are doing.  In order to keep track of your work it is good to name each of your columns clearly so that you will remember what they contain.  The structure for planing your workspace, which is going to be described below is not the only way of structuring your workspace, but it, seems to work.  Once you have the hang of the software you can alter the structure to suit your own needs and requirements.

One of the first things to remember is that computers are temperamental, particularly this SUN computer, so save your workspace regularly.  This is accomplished by using, [ip Window] [File: Save: Workspace].   This will open a save window similar to the one encountered during the image saving process.  Again simply give the workspace a name, followed by the suffix ‘.ws’, ensure you are saving in the right directory and then just save the workspace.  If you have problems saving your workspace refer to section 4.5, for more information.

NB: when you try to re-save a workspace with the same name as before the program will ask you if you want to overwrite the previous version.  In this situation you can normally just click ok.  If you have made major changes, which you are not sure of and do not want to delete the last workspace, you saved simply give the second version a different name.

#### Structuring Your Workspace:

Obviously the structure of your workspace will alter depending on the number of images you have to mosaic, but here the structure given will be for a mosaic comprised of 4 rows and 6 columns, a total of 24 images, see figure 2.

When the program starts up you will begin with one empty column.  If you or someone else, have already been working with the software it is good to save your previous work and then delete all of the existing columns and then open a fresh empty one to begin your mosaicing.

You should begin by loading images to you first column, but not all of them, in this case just the six images in the first row.  You can then re-name your column to be ‘R1-images’.  You then want to open a new column, before the column appears you will be presented with a small window in which you can click on the ‘comment’ line and give a name to the column.  Name this second column, ‘R1:1-3&4-6’ i.e. in this column you will be mosaicing two sets of three images, images 1-3 and then images 4-6.

1           2            3            4             5             6

R1

R2

R3

R4

gy

Figure 1: Diagram showing the numbering and positioning of the images in a 4 x 6 mosaic.  The size of one whole image, image R1.1, has been marked with diagonal lines.

Now with column ‘R1:1-3&4-6’  selected you then want to open up images 2 & 3 from R1 and assign your regions, then mosaic them together.  This will produce a total of three entries in your second column.  Once this has been done you want to open up the image produced from joining 2&3 and then mosaic it to image 1.  You will now have 6 entries in you column.  You can then repeat the process for images 4-6, i.e. mosaic 5 to 6 and then mosaic the resultant image to image 4.

When you are not using columns it is good to minimise then by CLMB on the black triangle in the top right corner of the column.  You can even move the minimised columns up into nice rows down the left side of your ‘ip window’, though this is up to you.

You can then repeat this process for each of the three other rows, producing columns named; ‘R2-images, R2:1-3&4-6, etc.

You know want to open another new column, call this one, ‘1-3:R1&R2’.  Now with this new column selected open up the mosaic of images 1-3 from R1 and images 1-3 from R2.  These two images should, ideally be the ninth entries in the rows named, ‘R1:1-3&4-6’ and ‘R2:1-3&4-6’, though you may have to check this.  The two images can then be mosaiced together, this will actually complete the upper left corner of the complete image.  Once this is done open up new columns and complete the other three corners of the painting, naming these columns, ‘4-6:R1&R2’, ‘1-3:R3&R4’, etc.

Once you have completed all of the four corners you can then open a final column and call it ‘Mosaic’.  With this column selected you then want mosaic the final image in ‘1-3:R1&R2’ with the final image in ‘1-3:R3&R4’, to produce the left half, and then similarly produce the right half.

The complete mosaic can then be produced by mosaicing the two new images you have produced in your ‘Mosaic’ column together.  This should produce your completed IR mosaic.  This column naming system will obviously need to be extended or altered depending on the number of image, but keep it clear and concise so that you can follow your own work and be able to make corrections were required.

### Problems with Mosaics:

- You have produced you mosaic but it looks all patchy, with squares of light and dark.

This should not be a problem, as it will be solved using the balancing commands, this will be noted below.

- There seems to be an error or errors in the image, normally seen down the edge of a painting but possibly in the centre.  This can be the result of two separate problems.

- Mosaicing problems: One or more of your joins, though they appeared ok before, are not quite correct.  This will probably be the case if you can see something similar to examples given in figure 3.

Figure 2: Schematic of some of the possible mosaicing problems, obviously the real problems may be more subtle that this.

To solve this type of problem you need to go back to your naming grid pattern, figure 2, and work out which images or group of images are wrong.  Once you have found this you need to go back to your workspace, find the correct column, and then the two regions you picked which resulted in the problem. DLMB on the buttons associated with these two regions, in order to open up the images.  You can then simply move the regions, by HLMB and dragging the region a new location.  NB: Each time you move a region the computer will recalculate all of the mosaics that it produced after that one, this may take some time, you will need to be patient.  You will probably have to move two regions, to correct each problem, though you may get away with just moving one of them.

- Mathematical Problems: This one is slightly harder to fix. The curvature of the IR tube, the angle of the painting on the easel or even the positioning of the camera stand, can lead to dimensional irregularities in the captured images.  When the computer joins images it does various mathematical things to the edges to achieve the join and this can magnify or build up some of the dimensional irregularities, as the images are mosaiced.  With this type of problem you can end up with slightly different problems, see figure 4.

Figure 2: Schematic of some of the type of problems caused by mathematical/dimensional errors, obviously the real problems may be more subtle that this.

A

B

This type of problem has been solved in previous cases by increasing the overlap, when mosaicing the larger section.  For example, if you were mosaicing 24 images, as shown in figure 2, using the workspace structuring suggested, problems A and B can be solved as follows.

Once you have produced all four, corner segments of the mosaic you will need to produce additional segments to overlap the problem join.  With ‘A’, in figure 4, you would also produce mosaics of  ‘1-3:R2&3’ and ‘4-6:R2&3’ and then join the respective corner segments on to the tops and bottom, to produce the two completed halves before you can join them to produce the completed mosaic.  Obviously with situation ‘B’ you would need to produce ‘3-4:R1&R2’ and ‘3-4:R3-R4’ instead.

NB: Remember, that if you did not capture the images yourself and you are not sure that the painting was completely flat during the capturing procedure it may not be possible to produce a perfect mosaic.  So don’t spends hours trying to solve a problem which you might not be able to fix.

### Balancing the contrast of the image:

Once you have a completed mosaic the last thing you have to do with IR software is to balance out all of the contrast differences that you may get from image to image.

- [ip window][Balance:Global_mosaic_balanch_search]

Again for large mosaics this will take a long time, as the computer is going back to the original images and then comparing them as they are put together and working out the overall contrast.

This is supposed to be completely automatic, though there are some problems with it.  The only repeatable error I have found is that if you have had to increase the overlaps so that you are actually using images more than once, as described above to solve the mathematical problems, the balancing will not currently work.  If this is the case then it can be balanced on the other Dell computer, with the linux version of the software or you can balance the smaller mosaiced section, save the smaller balanced images, [image window][File:save:image], and then start a new workspace with the balanced images and work your way to the finished product.  This error has been removed on the newer version of the software, but this is still to be installed tested etc.

If you are still alive and have produced a complete mosaic without killing yourself, a colleague or more likely the computer, WELL DONE.

### Converting your finished image:

Once you have your completed and saved image it will be in a vips format, i.e. *.v, in order for it to be printed out it needs to be converted to a ‘tif’ format.

Minimise or close your ‘ip window’ and then move the mouse pointer over to the ‘cmdtool’ window:

- Type pwd and press return this command will check which directory you are in, you can then use the ls command to check that your image was saved in this directory.

- If you need to change directory, type in cd /home/general/**1/**2, (where **1 is the name of your own directory and **2 is the name of the subdirectory in which you have saved your images), and press return.

- You can again check with ls to ensure your file is present.

When you are sure you are in the right directory you can continue as follows:

- Type in: im_vips2tiff *.v **.tif none and then press return.  (where * is the name of your image and ** is the name you want the new version of the image to be.  NB: these names can be the same if you want.

The computer will now have produced your tif image.

### Transferring your image and printing:

Your tif file then needs to be transferred to the PC in the lab to be tweaked with PhotoShop and then printed out.  This can be done by transferring image from the SUN to the networked PC, in the photographic studio, and then transferred to the lab on a Zip disk.   The exact ins and outs of this process is going to be changed soon so at the moment just grab Chris or Joe when you need it done.

## Continuing Previous Work:

In order to continue to mosaic a set of images, that you have already begun but not finished, you will need to reload in your workspace.  Turn on the computer and login as described before, open up or select the vips software widow and reload your previously saved workspace as follows:

- [ip Window] [File: Load: Workspace]

You can now continue on from were you left off mosaicing the rest of the image, as described in chapter 5.

## Process Summary:

##### Turning on the Computer:

- Plug the computer in and switch on the boxes in the correct order.

- Enter user name ‘general’ and the password ‘fun@sun’, when asked to.

##### Creating your own folders:

- Produce your own folder in General and then sub-directories or folders for each painting or project you examine.

- [file manager window][CRMB, File:Create Folder]

- Name your folder.

##### Opening the Infra-red imaging software:

- Open the ‘ip.binary.s26’ window, which runs the IR software.

- CRMB over background, then [Programs:Infrared Systems]

- Increase the size of the ‘ip window’ and then simplify the ruler, HRMB over blank section of ruler and then select ‘infrared’.

##### Capturing images:

- Add 2 video windows to column: [ip Window] [File: New: Video]

- Open both video windows, DLMB on video button.

- Ctrl-C, will capture new images to any video window you are using.

##### The camera is then positioned:

- Position stand and camera so that you can see the required amount of detail and the camera can be moved on the stand to cover the entire are of interest.

- Move the camera so that an image of the upper left corner of the painting or area of interest can be captured.

- Move the camera to the right two notches and capture a new image.

- Establish required overlap. Then close one of the video windows.

##### Preparation for Saving Images:

- Open up the save window for your current video window.

- [Video Window] [File: Save].

- Set path and file type and enter the name for the first image your are to save, for example, gyR1.1.v

- Ensure that the ‘Increment filename on save’ button has been selected.

##### Capture and save all of your images:

- Ctrl-C, and then saving, ensuring that you correct the file name at the start of each row.

- Capture and save your, grey card image, grey.v

- Then you can minimise the ‘ip window’.

##### Light correct your images:

- Move the mouse pointer over to the ‘cmdtool’ window:

- Type in cd /home/general/**1/**2, (where **1 is the name of your own directory and **2 is the name of the subdirectory in which you have saved your images), and press return.

- Use the ls command to check that your images are in this directory.

- Type in vips7.4 light_correct grey.v  *.v and then press return.  Please wait this can take a while if you have a lot of images.

- The computer will the produce your ic_* images.

##### Procedure for Mosaicing IR images:

- Turn on the computer as described in section 4.1.

- Open up the IR software as described in section 4.3.

##### Loading images:

- Images can be loaded into the software and added to a column using the following command:

- [ip Window] [File: Load: Image]

- The images can then be viewed by DLMB on the associated button.

- Entries I columns can be removed by selecting them and then pressing Ctrl-R.

#### Selecting and removing points for mosaicing:

- Open up the two images to be joined, examine them and then select the overlap region by CRMB.

- These selected points can be moved by HLMB over the point and dragging t to the new position.

- Regions can be removed by DLMB one the region and them choosing the remove option from the pop up menu.

#### Mosaicing:

- When you try to mosaic images the computer will always try to combine the last two entries in the selected column, unless you manual indicate which entries you want it to join together.

- [ip window][Mosaic: Left_right_mosaic]

- [ip window][Mosaic: Top_bottom_mosaic]

##### Workspaces and Their Structure:

- Take care with your workspace and save it often.

- [ip Window] [File: Save: Workspace].

##### Loading Workspace:

- [ip Window] [File: Load: Image]

##### Balancing the contrast of the image:

- Once you have a completed mosaic the last thing you have to do with IR software is to balance out all of the contrast differences that you may get from image to image.

- [ip window][Balance:Global_mosaic_balanch_search]

##### Saving your final image:

- Once you have opened up your completed image use the following command to save it.

- [image window][File:save:image]

##### Converting your File:

- Move the mouse pointer over to the ‘cmdtool’ window:

- Type in cd /home/general/**1/**2, (where **1 is the name of your own directory and **2 is the name of the subdirectory in which you have saved your images), and press return.

- Use the ls command to check that your images are in this directory.

- Type in: im_vips2tiff *.v **.tif none and then press return.  (where * is the name of your image and ** is the name you want the new version of the image to be.

## Common General Problems:

This is just a simple list of problems, which have been experienced by me, during the time I have been using this program.  Feel free to add additional irritations as you experience then. 

- Core Dumps and other Crashes: The software on the SUN computer is prone to crashing or core dumping, which is basically the same thing.  The program will shut down and often leave a large dumped file in your directory.  If this happens simply reload the IR software and carry on, open a video window and carry on capturing images or reload your workspace.

- The video window has been known to stop working, either showing garbled images, a black screen or simply not opening and showing you an error message.  In this case you can try closing the program and re-opening it but you will normally have to reboot the system.  If rebooting is required you will currently have to get Joe or Chris to do this.

- The IR software itself often just freezes up and does not seem to want to do anything.  In this case try to ‘quit’ it by CRMB on the small triangle in the top left corner of the window.  This may not do anything straight away, but you can simply CLMB on the same triangle to minimise it, then you can ignore it until it decides to close properly.  In the mean time you can simply open up a new copy of the program and carry on.

- Normally if possible the best solution is to save your work close the program and then start again.

- If you experience other problems feel free to stop and ask Joe or Chris.

- Good Luck .

## Shutting Down The Sun Computer:

When you have finished using the computer, NEVER just switch it off, it needs to be shutdown properly.  At this time only Joe and Chris can shut the sun computer down so leave it on a let one of them know.

The camera control, the three monitors and the IR source can all be switched off, however when unplugging the power leads make sure you do not unplug the computer until it has been properly shutdown.  If in doubt leave all of the plugs connected.

Again the computer can currently only be turned off by someone with the correct password, DO NOT TRY TO TURN IT OFF UNLESS YOU KNOW WHAT YOU ARE DOING.

Booting up: means that the computer will warm up and run the various programs that it needs to function.

Windows represent the interface between the computer and the user, either as a control area for a specific program or as an open window or command tool (cmdtool) through which commands can be given to the computer manually by typing them in.  Windows are normally square or rectangular boxes, which appear on the screen, with their name or title show across the top.  Windows of all sorts can normally be moved around the screen by HLMB over the uppermost section of the window, normally the title bar and then just moving the mouse.  Once you have moved the window to where you want it just let go of the mouse button.  Windows can be removed or minimised, (i.e. made smaller and move to the bottom of the screen), via the menu which appears when you HRMB over the small triangle in the top left-hand corner of the window.  Pick ‘quit’ from the menu to remove a window, or ‘close’ to minimise it.  These windows can also be minimised by CLMB on the same little triangle.  To reopen a minimised window just DLMB over the minimised window.

The background can be described as the tabletop or surface on to which the computer opens up various different programs.  The background of computer can be blank, patterned or even be made from a photograph or other scanned image.

The ‘cmdtool’ or command tool window allows you to type in specific commands and instructions directly to the computer.

The ‘cmdtool (console)’ just acts as a notice board were the computer leaves messages to inform you when certain commands or function have taken place.  You do not really need to interact with this window at this level.

The information stored on a computer is like an electronic version of a filing system.  With computers spaces in the computers memory are defined as directories or folders and then actual data, information commands etc. are defined as files.  By creating directories your are simply defining an area in which you can store files.  It should be noted that directories work almost like a tree; you will have a root directory, in which all the information on the computer is housed.  Then within this directory there will be subdirectories and then again subdirectories within the subdirectories, just like the branches of the tree.  The program File Manager simply supplies a graphical representation of the computers filing system, through which you can move in and out of directories examine files and create new directories.

Ruler: The ruler on a window is the upper section that contains commands, information and sometimes a scale.

All files are given names so that they can be distinguished from one another.  In the file manager the name of a directory or file will appear under the graphical image of the file or directory.

Path: the path indicates which part of the computers filing system is open.

To change the size of a window move the mouse arrow to the bottom right corner of the window, HLMB and move the mouse around this will alter the size and shape of the window.  Once the window is the desired size let go of the mouse button.

When the program first opens there will be no entries within the column so do not get upset if you can not see any.

These buttons appear as slightly raised rectangles down the left of the column and each is given a letter and a number.  The letter is specific to the column and the number just indicates the order in which the entries have been opened, i.e. the first entry in column A will be numbered ‘A1’, etc.

If you have more than one column open the selected one will be the one with the darkened title bar.  You can select different columns by CLMB over the title bar of the different columns.

To change the size of a window move the mouse arrow to the bottom right corner of the window, HLMB and move the mouse around this will alter the size and shape of the window.  Once the window is the desired size let go of the mouse button.

Sliders are thin bars, which can appear at the left and bottom edges of windows, they normally have a small arrow at each end by CLMB on either of these you can move the image around.   If the sliders are not present then the window is currently showing you the entire image.

When dealing with computers the ‘*’ symbol is often used as shorthand to describe several objects or names rather than specific ones.

To counteract the problem of uneven sensitivity across the target area of the tube, it is necessary to capture an image of a piece of grey card. This grey card image is then used to correct all subsequent images. The card should be a flat, even grey, of around 50% reflectance.  The Vidicon and lighting positions with respect to the painting should not be changed between the imaging of the grey card and the capturing of the mosaic. A grey card can be captured at any time, but it is good practice to start with one | you may forget later.  If your mosaic will take several hours to capture, you may wish to grab extra grey cards, since the sensitivity of the tube can change a little as it warms up. Be sure to note which data images correspond to which grey cards!

Columns can be renamed at any time simply by DLMB on the top section of the column, typing in  the new name and then pressing return.

Each of the images, you will be examining, are composed of little dots.  The computer counts the little dots and uses them as a way of navigating around the images, i.e. a certain point may be 235 points up and 1024 points in from the edge.  You can seen the co-ordinate numbers being displayed by the computer, as you move the mouse cursor over an image, at the top left of the image viewing window, in the format (123 x 34).  The third number is related to how light or dark each particular area is.

This will indicate that, from the ip Window ruler bar you should CLMB once on the ‘File’ heading and then CLMB once on the ‘save’ entry in the drop down menu and then finally CLMB once on the ‘workspace’ entry in the sub menu.

It is very good practice to delete/remove any extra regions/images, which may be produced by accident or through experimentation, in order to keep your columns as straight forward as possible.  NB: If you remove entries from columns which are involved with the mosaicing of other entries, either in the same column or in different columns you will start to produce error messages and things will get confusing, so be careful.

Format here just denotes the type of file, images can be stored on computer in various ways the most common being, JPEG, tif, and bitmap.  You do not really have to worry about this though.

Workspaces are used during the mosaicing procedure and will be discussed in section 5.